# OMDOAO3 README File

Pepijn Veefkind, KNMI

Last update: 9 October 2007

## Overview

This document provides a brief description of the OMDOAO3 data product. The OMDOAO3 Level 2 data product contains total ozone and ancillary information produced by the OMI DOAS algorithm. Each file contains the sunlit part of an OMI orbit, from the South Pole to the North Pole. In the so-called global observation mode, the OMI swath is approximately 2600 wide, providing daily global coverage

The OMDOAO3 is produced as a standard product as well as in near-real-time (within three hours of observation) and as a very fast delivery product (within 20 minutes of observation). Although it is expected that the differences will be very small, this document describes the standard data product. The near-real-time products is available from the TEMIS web site and the very fast delivery product from the FMI web site.

You may refer to release specific information about OMDOA3 for details about software versions and known problems.

## Algorithm Description

The OMI DOAS algorithm is described in the OMI Algorithm Theoretical Basis Document (ATBD). In addition, a paper that contains a description of the algorithm has been published in IEEE Transactions on Geoscience and Remote Sensing (IEEE TGRS, Vol 44, No. 5, 2006, doi: 10.1109/TGRS.2006.871204).

In the DOAS (Differential Optical Absorption Spectroscopy) implementation for OMI the ozone vertical column is determined in three steps. In the first step the actual DOAS fitting is performed, resulting in the so-called slant column density, which is the amount of ozone along an average photon path from the Sun -through the atmosphere- to the satellite. In the second step the air mass factor is determined, which is needed to convert the slant column density into a vertical column. In the last step a correction is performed for clouds. All the results from intermediate steps are also contained in the product.

The OMI DOAS spectral fitting uses a 5 nm wide fit window centered around 334.1 nm. This window has been selected based on the very low temperature sensitivity for this window. In the spectral fitting, the inelastic rotational Raman scattering, as well as the effective ozone temperature are explicitly accounted for. The air mass factor is determined off-line, by simulating OMI spectra using a radiative transfer model. The cloud pressure needed to derive accurate air mass factors and for correction factors for cloudy and partly cloudy conditions, is obtained from the OMCLDO2 cloud product. This product derives the cloud fraction and pressure from the O2-O2 absorption band around 477 nm. The OMI Algorithm Theoretical Basis Document (ATBD) and the above mentioned paper in IEEE TGRS contain a complete description of the algorithm. The effective cloud fraction is derived from the fit window itself.

This algorithm is one of the two algorithms that will be used to derive total ozone values from OMI. The other is an algorithm based on the well-known TOMS method (OMTO3 data product). Initial comparisons show good overall agreement between the two algorithms, but there are noticeable differences over clouds and snow/ice, as well as at large solar zenith angles.

## Data Quality Assessment

In this section the general sata quality is discussed. The release specific information about OMDOA3 contains details on specific features and problems in the data product and this document should be read before using the data.

Validation was performed by comparing the OMDOAO3 total ozone data version 1.0.1  with collocated Brewer data for the Northern hemisphere, for November 2004 (see above mentioned paper in IEEE TGRS) as well as for the period between March 28 and June 11, 2005. On average, the retrieved ozone column is biased by 2 % with respect to the Brewers ([OMI-Brewer]/Brewer), the standard deviation on this number is 3.5%. The validation results show that the errors increase with increasing Solar zenith angle. In the current version of OMDOAO3 (1.0.5) the Solar zenith angle dependent bias is expected to be much smaller. This is confirmed by comparing to the OMTO3 Collection 2 dataset.

Several papers on the validation of the OMDAO3 data product v1.0.1 will be part of a special issue of JGR on the validation of Aura data products, to appear early 2008. We will continue to work on the validation of the Collection 3 version 1.0.5 data.

To assess the quality of the individual retrievals it is very important to lok at the quality flags fields in the data products, as described in the Product Format Specification. Especially the ProcesingQualityFlags are important to filter for bad quality data. The best quality data have a ProcesingQualityFlags of 0. It is recommended to apply a bitwise AND on the ProcesingQualityFlags field using a  value of 10911 to filter the data. This will filter all data for which the ProcesingQualityFlags bit 0, 1, 2, 3, 4, 7, 9, 11 and/or 13 are set.

## Product Description

A single OMDOAO3 product file contains all OMI measurements on the sunlit portion of the Earth, for one Aura orbit. During one orbit OMI performs approximately 1650 measurements, which take 2 seconds each. In the global observation mode, 60 across track ground pixels are measured simultaneously during a measurement. These 60 measurements cover a swath of approximately 2600 km wide. During so-called zoom-in measurements the swath width maybe reduced. The operational baseline includes zoom-in measurements for 1 day a month. In case of zoom-in measurements 30 of the 60 across track pixels contain data, the other 30 contain fill values.

A complete description of the format of the OMDOAO3 data products is provided here. A more condensed listing of the data fields is available in the Product Format Specification.

The OMDOAO3 product is written as an HDF-EOS5 swath file. For a list of tools that read HDF-EOS5 data files, please visit this link: http://disc.gsfc.nasa.gov/Aura/tools.shtml.

## Data Availability

The OMDOAO3 product can be obtained from the NASA Goddard Earth Sciences Data and Information Service Center (DISC). Full OMDOAO3 data, as well as subsets of these data over many ground stations and along Aura validation aircraft flights paths are also available through the Aura Validation Data Center (AVDC) website) to those investigators who are associated with the various Aura science teams. Bojan Bojkov is the point of contact at the AVDC.

For users not interested in the detailed information provided on OMDOAO3 dataset we are developing several gridded products. In addition, we intend to make OMDOAO3 data available in a geographically ordered (rather than time-ordered) format that can be more easily subsetted and manipulated on-line prior to ordering. Please check the Goddard Earth Sciences (GES) Data and Information Service Center for current information on these products.

## Contact Information

Questions related to the OMDOAO3 dataset should be directed to the GES DISC. For questions and comments related to the OMDOAO3 algorithm and data quality please send mail to contact omdoao3@ltpmail.gsfc.nasa.gov.